Salient Features

ARC Flash Protective Gear

(All Kits Tested/Certified prior to shipping, within 180 days prior to delivery to customer)

- 22 (twenty-two) 40 cal/sq cm (PPE Category IV) Kits*:

* 40 cal/sq cm Kits, at a minimum, shall include adequate protective gear; safety glasses, leather glove protectors, 14 inch (between 14 inch-16 inch) rubber insulated gloves (class 2), jacket, hood with hard cap, bib overalls, gear bag, glove bag.  Additional items may be included with Kits.

4 Ea ARC Flash protection clothing kit (size Large): 40 cal/sq cm, Gloves (size 8);

4 Ea ARC Flash protection clothing kit (size Large): 40 cal/sq cm, Gloves (size 9);

2 Ea ARC Flash protection clothing kit (size Large): 40 cal/sq cm, Gloves (size 10);

5 Ea ARC Flash protection clothing kit (size X-Large): 40 cal/sq cm, Gloves (size 10);

1 Ea ARC Flash protection clothing kit (size Small): 40 cal/sq cm, Gloves (size 8);

2 Ea ARC Flash protection clothing kit (size X-Large): 40 cal/sq cm, Gloves (size 11);

2 Ea ARC Flash protection clothing kit (size Medium): 40 cal/sq cm, Gloves (size 8);

1 Ea ARC Flash protection clothing kit (size 2X-Large): 40 cal/sq cm, Gloves (size 9);

1 Ea ARC Flash protection clothing kit (size Medium): 40 cal/sq cm, Gloves (size 9);

_____________________________________________________________________________________

- 42 (forty-two) 8 cal/sq cm (PPE Category II) Kits*:

* 8cal/sq cm Kits, at a minimum, shall include adequate protective gear; balaclava, face shield, face shield bag, hard hat, coverall, storage bag.  Additional items may be included with Kits.

1 Ea ARC Flash clothing kit (size 2X-Large): 8cal/sq cm

11 Ea ARC Flash clothing kit (size X-Large): 8cal/sq cm

22 Ea ARC Flash clothing kit (size Large): 8cal/sq cm

7 Ea ARC Flash clothing kit (size Medium): 8cal/sq cm

1 Ea ARC Flash clothing kit (size Small): 8cal/sq cm

_____________________________________________________________________________________

- 42 (forty-two) Class 0 Glove (PPE Category III) Kits*:

* Class 0 Glove Kits, at a minimum, shall include adequate protective gear; 11 inch (between 10 inch - 12 inch) rubber insulated gloves, leather glove protectors, glove bag.  Additional items may be included with Kits.

18 Ea Electrical-insulating glove kit: 1,000AC/1,500DC/Gloves (size 8)

12 Ea Electrical-insulating glove kit: 1,000AC/1,500DC/ Gloves (size 9)

10 Ea Electrical-insulating glove kit: 1,000AC/1,500DC/ Gloves (size 10)

2 Ea Electrical-insulating glove kit: 1,000AC/1,500DC/ Gloves (size 11)